ADST11SD300&310

Fingerprint Identification Module

User’s Guide

(Ver 4.0)

ID System Technology Co.,Ltd

http://www.szadst.com

Tel:86-755-33160920

Fax:86-755-86039586

List

1. Summary	3

1.1 Main functions	3

1.2 Application examples	3

1.3 Technology Parameter	4

1.4 Default Setting	4

2. Configration and Connector	5

2.1 Mount and Connector	5

2.2 Dimension of ADST11SD300	6

2.3 Dimension of ADST11SD310	7

3. Communication Protocol	7

3.1 The process of communication	7

3.2 Classify of communication packet	8

3.2.1 Command packet	8

3.2.2 Response packet	8

3.2.3 Data Packet	8

3.3  Packet Structure	9

3.3.1 Packet Identify Code	9

3.3.2 Structure of Command Packet	9

3.3.3 Response packet	9

3.3.4 Command Data Packet	10

3.3.5 Response Data Packet	10

4. Packet Command Detail Description	11

4.1 Structure of Fingerprint Template Data is follow	11

4.2  Command List	11

5. Appendix	14

5.1 Response and Error Code	14

5.2 Flow of Enroll and Verify and Identify process	15

5.2.1 Enroll	15

5.2.2 Verify & Identify	16

# 1. Summary

Functions and Characteristic:

ADST11SD3xx serial is All-in-one (Optic sensor +CMOS image sensor + CPU +Flash memory) embedded stand-alone fingerprint identification module, it is convenient for second development,Extremely easy to integrate minimizing time-to-market.

1.1 Main functions

Optic sensor is reliable and Low-cost, High ESD Protection

1:N Identification (One-to-Many)

1:1 Verification (One-to-One)

High speed fingerprint identification algorithm engine

Self study function

Fingerprint template data read from /write to FLASH memory function

Get Feature Data of Captured fingerprint

Verify Downloaded Feature with Captured fingerprint

Identify Downloaded Feature with Captured fingerprint

Security Level setting

Standby mode

Communication interface :UART

1.2 Application examples

- Access control systems

- Time & Attendance

- Locks, safes

- POS terminals

1.3 Technology Parameter

| Item | Description |
| --- | --- |
| CPU | Core:ARC (120Mhz,1M bit Flash Memory,128K SRAM) |
| Sensor | Optic sensor |
| Fingerprint capacity | 500~5000 can be enlarge，Default setting is 3000 fingerprints |
| FAR | < 0.001 %（Security Level is 3） |
| FRR | < 0.1 %（Security Level is 3） |
| Match Mode | 1:N identification and 1:1 verification |
| Fingerprint Data size | 496 Byte |
| Security level | Level 1 to Level 5 can be set , default setting is : Level 3 |
| Time of Response | Fingerprint pretreatment < 0.45 |
|  | 1:N matching（3000 fingerprints）< 1s |
| Resistant to Abrasion | >1 Million Finger Press |
| Communication | UART：Parity = NONE,Stop Bit = 1,Flow Control = NONE Baud rate is 9600/19200/38400/57600/115200 (BPS) Default setting：115200bps |
| Working Voltage | DC 3.3V-5V |
| Working Current | Max 100mA，Standby Mode less than 5mA |
| Working Environment | Working Temperature：-10℃ to 60℃ Relative humidity：20%– 80% |
| Storage Environment | Storage temperature: -20℃ to 80℃ Storage humidity:<90% |

1.4 Default Setting

| Item | Value |
| --- | --- |
| Security Level（1～5） | 3 |
| Finger Time Out（0～60S） | 5s |
| BaudRate（9600，19200，38400，57600，115200）BPS | 115200 BPS |
| Fingerprint Duplication Check（ON/OFF） | ON |

# 2. Configration and Connector

2.1 Mount and Connector

Fig2-1. Shape of ADST11SD300&310

Table2-1. Connector Signal description

| Pin No. | Color | Signal name | Signal Description |
| --- | --- | --- | --- |
| PIN 1 | White | UART-TX | Module Transmit Output,3.3V TTL Logic |
| PIN 2 | Yellow | UART-RX | Module Receive Input,3.3V TTL Logic |
| PIN 3 | Black | GND | GND |
| PIN 4 | Red | VIN | DC Input,Range of voltage is 3.3V |

2.2 Dimension of ADST11SD300

Fig2-2. Dimension of ADST11SD300

2.3 Dimension of ADST11SD310

Fig 2-3. Dimension of ADST11SD310

# 3. Communication Protocol

The module is used as a slave device. The Master device sends relative commands to control it.

The CMD sent by the master and the ACK signal returned by the module.

Command interface: UART (Universal Asynchronous Receiver Transmitter)

Default Setting is : 115200BPS ,1 start-bit ,1 stop-bit ,no check bit

3.1 The process of communication

H O S T                                 T A R G E T（ADST11SD300&310）

Begin

Wait Command

Command Packet

Send Command

Receive Command

Data Packet

Process Command

Response Packet

Receive Result

Send Result

Data Packet

End

Fig3-1.Process of Communication

Notice. Host do not send next command until receive result , except send FP Cancel

3.2 Classify of communication packet

3.2.1 Command packet

Command Packet is the instruction from Host to Target (ADST11SD3xx),

Total length of the command packet is 24 Bytes

3.2.2 Response packet

Response packet is result of execute command packet, from Target (ADST11SD3xx) to Host,

Total length of the command packet is 24 Bytes

3.2.3 Data Packet

When length of Command Parameter or Data is larger than 16 Bytes, Utilize Data Packet

to transmit block Data, the maximum length of Data Packet is 512Bytes

3.3  Packet Structure

3.3.1 Packet Identify Code

Section of start 2byte prefix define type of packet

| Type of Packet | Code |
| --- | --- |
| Command packet | 0xAA55 |
| Response packet | 0x55AA |
| Command Data Packet | 0xA55A |
| Response Data Packet | 0x5AA5 |

3.3.2 Structure of Command Packet

| PREFIX |  | CMD |  | LEN |  | DATA |  |  |  | CKS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | L | H | L | H | D0 | D1 | … | D15 | L | H |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | … | 21 | 22 | 23 |

| OFFSET | FIELD | TYPE | SIZE | DESCRYPTION |
| --- | --- | --- | --- | --- |
| 0 | PREFIX | WORD | 2byte | Packet Identify code |
| 2 | CMD | WORD | 2byte | Command Code |
| 4 | LEN | WORD | 2byte (=n, n < 17) | Length of Command Parameter |
| 6 | DATA | Byte array | 16byte | Command Parameter |
| 22 | CKS | WORD | 2byte | Check Sum is the low word of value listed below: offset[0] + offset[1] + …+offset[21] |

3.3.3 Response packet

| PREFIX |  | RCM |  | LEN |  | RET |  | DATA |  |  |  | CKS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xAA | 0x55 | L | H | L | H | L | H | D0 | D1 | … | D13 | L | H |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | … | 21 | 22 | 23 |

| OFFSET | FIELD | TYPE | SIZE | DESCRYPTION |
| --- | --- | --- | --- | --- |
| 0 | PREFIX | WORD | 2byte | Packet Identify code |
| 2 | RCM | WORD | 2byte | Response Code |
| 4 | LEN | WORD | 2byte(=n, n < 17) | RET and DATA |
| 6 | RET | WORD | 2byte | Result Code (0 :success，1 :fail） |
| 8 | DATA | Byte array | 14byte | Response Data (n-2 byte) |
| 22 | CKS | WORD | 2byte | Check Sum is the low word of value listed below: offset[0] + offset[1] + …+offset[21] |

3.3.4 Command Data Packet

| PREFIX |  | CMD |  | LEN |  | DATA |  |  |  | CKS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x5A | 0xA5 | L | H | L | H | D0 | D1 | … | Dn-1 | L | H |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | … | 6+n-1 | 6+n | 6+n+1 |

| OFFSET | FIELD | TYPE | SIZE | DESCRYPTION |
| --- | --- | --- | --- | --- |
| 0 | PREFIX | WORD | 2byte | Packet Identify code |
| 2 | CMD | WORD | 2byte | Command Code |
| 4 | LEN | WORD | 2byte(=n , n < 512) | Length of DATA |
| 6 | DATA | Byte array | nbyte | Command parameter |
| 6+n | CKS | WORD | 2byte | Check Sum is the low word of value listed below: offset[0] + offset[1] + …+offset[6+n-1] |

Before send Command Data packet, Host first send Command packet which set the length of next command data packet in Data Field

3.3.5 Response Data Packet

| PREFIX |  | RCM |  | LEN |  | RET |  | DATA |  |  |  | CKS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA5 | 0x5A | L | H | L | H | L | H | D0 | D1 | … | Dn-3 | L | H |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | … | 6+n-1 | 6+n | 6+n+1 |

| OFFSET | FIELD | TYPE | SIZE | DESCRYPTION |
| --- | --- | --- | --- | --- |
| 0 | PREFIX | WORD | 2byte | Packet Identify code |
| 2 | RCM | WORD | 2byte | Response Code |
| 4 | LEN | WORD | 2byte(=n, n < 512) | Length of result data：(RET + DATA) |
| 6 | RET | WORD | 2byte | Result code （0 : success 1 : fail） |
| 8 | DATA | Byte array | （n-2）byte | Response data |
| 6+n | CKS | WORD | 2byte | Check Sum is the low word of value listed below: offset[0] + offset[1] + …+offset[6+n-1] |

# 4. Packet Command Detail Description

4.1 Structure of Fingerprint Template Data is follow

Template Data(496Bytes)+CheckSum(2Bytes)=498Bytes

| Template Data | CheckSum |
| --- | --- |
| 496 bytes Template Data D0,D1,…D495 | 2 byte Check Sum is the low word of value listed below: D0+D1+…+D495 |

4.2  Command List

| No | Function | Command |  |  | Response |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CMD | LEN | DATA | RCM | LEN | RET | DATA |
| 1 | Verify | 0x0101 | 2 | Template No. | 0x0101 | 4 | 0/1 | Template No./ Error Code |
| 2 | Identify | 0x0102 | 0 | - | 0x0102 | 4 | 0/1 | Template No ./ Error Code |
| 3 | Enroll | 0x0103 | 2 | Template No. | 0x0103 | 4/6 | 0/1 | Template No. / Error Code + Duplicaion ID |
| 4 | Enroll One Time | 0x0104 | 2 | Template No. | 0x0104 | 4/6 | 0/1 | Template No. / ErrorCode |
| 5 | Clear Template | 0x0105 | 2 | Template No. | 0x0105 | 4 | 0/1 | Template No. / ErrorCode |
| 6 | Clear All Template | 0x0106 | 0 | - | 0x0106 | 4 | 0/1 | Cleared TemplateCount / Error Code |
| 7 | Get Empty ID | 0x0107 | 0 | - | 0x0107 | 4 | 0/1 | Template No. / Error Code |
| 8 | Get Template Status | 0x0108 | 2 | Template No. | 0x0108 | 4 | 0/1 | Template Status /Error Code |
| 9 | Get Broken Template | 0x0109 | 0 | - | 0x0109 | 6 | 0/1 | Broken Template Count + First Broken Template No./Error Code |
| 10 | Read Template | 0x010A | 2 | Template No. | 0x010A | ( Template Record Size + 4) or (4) | 0/1 | Template No + Template Record Data /Error Code |
| 11 | Write Template | 0x010B | Template Record Size + 2 | Template No. + Template Record Data | 0x010B | 4 | 0/1 | Template No. / Error Code |
| 12 | Set Security Level | 0x010C | 2 | Security Level Value | 0x010C | 4 | 0/1 | Security Level Value/Error Code |
| 13 | Get Security Level | 0x010D | 0 | - | 0x010D | 4 | 0 | Security Level Value |
| 14 | Set Finger Time Out | 0x010E | 2 | Time Out Value | 0x010E | 4 | 0/1 | TimeOut Value / Error Code |
| 15 | Get Finger Time Out | 0x010F | 0 | - | 0x010F | 4 | 0 | TimeOut Value |
| 16 | Set Device ID | 0x0110 | 2 | Device ID | 0x0110 | 4 | 0 | Device ID |
| 17 | Get Device ID | 0x0111 | 0 | - | 0x0111 | 4 | 0/1 | Device ID / Error Code |
| 18 | Get F/W Version | 0x0112 | 0 | - | 0x0112 | 4 | 0 | F/W Version |
| 19 | Finger Detect | 0x0113 | 0 | - | 0x0113 | 4 | 0 | Detect Result |
| 20 | Set BaudRate | 0x0114 | 2 | BaudRate Index | 0x0114 | 4 | 0/1 | BaudRate Index / Error Code |
| 21 | Set Duplication Check | 0x0115 | 2 | Duplication Check Option(1/0) | 0x0115 | 4 | 0/1 | Duplication Check Option /Error Code |
| 22 | Get Duplication Check | 0x0116 | 0 | - | 0x0116 | 4 | 0 | Duplication Check Option |
| 23 | Enter Stadby Mode | 0x0117 | 0 | - | 0x0117 | 4 | 0 | - |
| 24 | Enroll And Store in RAM | 0x0118 | 0 | - | 0x0118 | 4 | 0/1 | 0 / ErrorCode |
| 25 | Get Enroll Data | 0x0119 | 0 | - | 0x0119 | ( Template Record Size + 2) or (4) | 0/1 | Template Record Data /Error Code |
| 26 | Get Feature Data of Captured FP | 0x011A | 0 | - | 0x011A | ( Template Record Size + 2) or (4) | 0/1 | Template Record Data /Error Code |
| 27 | Verify Downloaded Feature with Captured FP | 0x011B | Template Record Size | Template Record Data | 0x011B | 4 | 0/1 | 0/Error Code |
| 28 | Identify Downloaded Feature with Captured FP | 0x011C | Template Record Size + 2 | Index + Template Record Data | 0x011C | 4 | 0/1 | 0/Error Code |
| 29 | Get Device Name | 0x0121 | 0 | - | 0x0121 | 16 | 0/1 | “STO20-OEM” /Error Code |
| 30 | Sensor LED Control | 0x0124 | 2 | 0/1 | 0x0124 | 4 | 0 | 0 |
| 31 | Identify Free | 0x0125 | 0 | - | 0x0125 | 4 | 0/1 | Template No. / Error Code |
| 32 | Set Device Password | 0x0126 | 14 | Password | 0x0126 | 4 | 0/1 | 0/ Error Code |
| 33 | Verify Device Password | 0x0127 | 14 | Password | 0x0127 | 4 | 0/1 | 0/ Error Code |
| 34 | Get Enroll Count | 0x0128 | 0 |  | 0x0128 | 4 | 0/1 | Enroll Count /Error Code |
| 35 | FP Cancel | 0x0130 | 0 | - | 0x0130 | 2 | 0 | - |
| 36 | Test Connection | 0x0150 | 0 | - | 0x0150 | 2 | 0 | - |
| 37 | Incorrect Command | - | - | - | 0x0160 | 2 | 0 | - |

Table4-2 Command List

# 5. Appendix

5.1 Response and Error Code

| No. | Response and Error Code | Value | Description |
| --- | --- | --- | --- |
| 1 | ERR_SUCCESS | 0x00 | Successful for command execute |
| 2 | ERR_FAIL | 0x01 | Fail for command execute |
| 3 | ERR_VERIFY | 0x11 | One to one match fail |
| 4 | ERR_IDENTIFY | 0x12 | Identify fail, no matched template |
| 5 | ERR_TMPL_EMPTY | 0x13 | The appointed Template Data is Null |
| 6 | ERR_TMPL_NOT_EMPTY | 0x14 | Existed Template Data for the appointed ID |
| 7 | ERR_ALL_TMPL_EMPTY | 0x15 | All template is Null, Without enrolled Template |
| 8 | ERR_EMPTY_ID_NOEXIST | 0x16 | Without available Template ID. |
| 9 | ERR_BROKEN_ID_NOEXIST | 0x17 | Without damaged Template. |
| 10 | ERR_INVALID_TMPL_DATA | 0x18 | Appointed Template Data is invalid. |
| 11 | ERR_DUPLICATION_ID | 0x19 | The fingerprint has been enrolled. |
| 12 | ERR_BAD_QUALITY | 0x21 | Bad quality fingerprint image. |
| 13 | ERR_TIME_OUT | 0x23 | During Time Out period, no finger is detected |
| 14 | ERR_NOT_AUTHORIZED | 0x24 | Command is without authorization |
| 15 | ERR_GENERALIZE | 0x30 | Generalize Template Data fail |
| 16 | ERR_FP_CANCEL | 0x41 | Relative fingerprint command have been cancelled |
| 17 | ERR_INTERNAL | 0x50 | Internal Error |
| 18 | ERR_MEMORY | 0x51 | Memory Error |
| 19 | ERR_EXCEPTION | 0x52 | Firmware Error |
| 20 | ERR_INVALID_TMPL_NO | 0x60 | Template No. is invalid |
| 21 | ERR_INVALID_SEC_VAL | 0x61 | The Value of Security Level is invalid |
| 22 | ERR_INVALID_TIME_OUT | 0x62 | The Value of Time Out is invalid |
| 23 | ERR_INVALID_BAUDRATE | 0x63 | The Value of Baud Rate is invalid |
| 24 | ERR_DEVICE_ID_EMPTY | 0x64 | Not setting Device ID |
| 25 | ERR_INVALID_ DUP_VAL | 0x65 | Option Value of Duplication Check is invalid |
| 26 | ERR_INVALID_PARAM | 0x70 | Parameter is invalid |
| 27 | ERR_NO_RELEASE | 0x71 | In process of IdentifyFree command, Finger is not release |
| 28 | GD_DOWNLOAD_SUCCESS | 0xA1 | Download Template Record Data is successful |
| 29 | GD_NEED_FIRST_SWEEP | 0xFFF1 | Waiting input fingerprint for the first time. |
| 30 | GD_NEED_SECOND_SWEEP | 0xFFF2 | Waiting input fingerprint for the second time. |
| 31 | GD_NEED_THIRD_SWEEP | 0xFFF3 | Waiting input fingerprint for the third time. |
| 32 | GD_NEED_RELEASE_FINGER | 0xFFF4 | Lift finger |
| 33 | GD_DETECT_FINGER | 0x01 | There is an finger on sensor |
| 34 | GD_NO_DETECT_FINGER | 0x00 | There is no finger on sensor |
| 35 | GD_TEMPLATE_NOT_EMPTY | 0x01 | The appointed Template Data are not null |
| 36 | GD_TEMPLATE_EMPTY | 0x00 | The appointed Template Data are null |

5.2 Flow of Enroll and Verify and Identify process

5.2.1 Enroll

No

No

No

Yes

Start

Capture Success?

Time Out ?

Waiting for finger

Yes

FP Cancel?

End

Result = Success

Yes

Result = Fail

Generalize Success?

Increasing Counter

Counter = 3 ?

Counter = 1

No

Yes

Yes

No

Fig 5-1 Flow of Enroll

5.2.2 Verify & Identify

Start

FP Cancel?

OK?

Verify or Identify

End

No

Yes

Yes

Result = Timeout

Result = Accept

Result = Reject

Time Out?

Result = Fail

Capture  Success?

No

Yes

Waiting for finger

No

Yes

No

Fig 5-2 Flow of Verify and Identify